`timescale 1ns / 1ps

module tb_alu_4bit;

reg [3:0] a, b;

reg [2:0] alu_sel;

wire [3:0] alu_out;

wire zero, negative, overflow;

integer pass_count = 0;

integer fail_count = 0;

// Device Under Test

alu_4bit uut (

.a(a),

.b(b),

.alu_sel(alu_sel),

.alu_out(alu_out),

.zero(zero),

.negative(negative),

.overflow(overflow)

);

task check;

input [3:0] ta, tb;

input [2:0] tsel;

input [3:0] expected_out;

input expected_z;

input expected_n;

input expected_v;

begin

a = ta;

b = tb;

alu_sel = tsel;

#10;

if ((alu_out === expected_out) &&

(zero === expected_z) &&

(negative === expected_n) &&

(overflow === expected_v)) begin

pass_count = pass_count + 1;

$display(

"PASS: sel=%b a=%b b=%b out=%b Z=%b N=%b V=%b",

alu_sel, a, b, alu_out,

zero, negative, overflow

);

end else begin

fail_count = fail_count + 1;

$display(

"FAIL: sel=%b a=%b b=%b",

alu_sel, a, b

);

$display(

"Expected: out=%b Z=%b N=%b V=%b",

expected_out,

expected_z,

expected_n,

expected_v

);

$display(

"Actual: out=%b Z=%b N=%b V=%b",

alu_out,

zero,

negative,

overflow

);

end

end

endtask

initial begin

$display("===== ALU 4-BIT TEST =====");

// ADD

check(4'b0011,4'b0010,3'b000,4'b0101,0,0,0);

check(4'b0000,4'b0000,3'b000,4'b0000,1,0,0);

check(4'b0111,4'b0001,3'b000,4'b1000,0,1,1);

check(4'b1111,4'b1111,3'b000,4'b1110,0,1,0);

check(4'b1000,4'b1000,3'b000,4'b0000,1,0,1);

// SUB

check(4'b0101,4'b0010,3'b001,4'b0011,0,0,0);

check(4'b0011,4'b0011,3'b001,4'b0000,1,0,0);

check(4'b1000,4'b0001,3'b001,4'b0111,0,0,1);

check(4'b0010,4'b0101,3'b001,4'b1101,0,1,0);

check(4'b0111,4'b1111,3'b001,4'b1000,0,1,1);

// AND

check(4'b1010,4'b1100,3'b010,4'b1000,0,1,0);

check(4'b1010,4'b0101,3'b010,4'b0000,1,0,0);

check(4'b1111,4'b0011,3'b010,4'b0011,0,0,0);

// OR

check(4'b1010,4'b0101,3'b011,4'b1111,0,1,0);

check(4'b1000,4'b0001,3'b011,4'b1001,0,1,0);

check(4'b0000,4'b0000,3'b011,4'b0000,1,0,0);

// XOR

check(4'b1010,4'b1100,3'b100,4'b0110,0,0,0);

check(4'b1111,4'b1111,3'b100,4'b0000,1,0,0);

check(4'b1000,4'b0001,3'b100,4'b1001,0,1,0);

// SHIFT LEFT

check(4'b1001,4'b0000,3'b101,4'b0010,0,0,0);

check(4'b1111,4'b0000,3'b101,4'b1110,0,1,0);

check(4'b1000,4'b0000,3'b101,4'b0000,1,0,0);

// SHIFT RIGHT

check(4'b1001,4'b0000,3'b110,4'b0100,0,0,0);

check(4'b1111,4'b0000,3'b110,4'b0111,0,0,0);

check(4'b1000,4'b0000,3'b110,4'b0100,0,0,0);

// UNSIGNED COMPARE

check(4'b0010,4'b0101,3'b111,4'b0001,0,0,0);

check(4'b0101,4'b0101,3'b111,4'b0000,1,0,0);

check(4'b1000,4'b0111,3'b111,4'b0000,1,0,0);

$display("=========================");

$display("PASS = %0d", pass_count);

$display("FAIL = %0d", fail_count);

$display("=========================");

$finish;

end

endmodule